Pedestrian Detection System for Automotive Safety Applications

# Pedestrian Detection System using YOLO and IMX500 #

Project Overview

This project implements a pedestrian detection system using a lightweight YOLO-based object detection model.

The system is designed for automotive safety applications, such as detecting pedestrians near zebra crossings.

The trained model is optimized for embedded deployment on the Sony IMX500 AI vision sensor and can also be tested in a desktop environment.

Dataset

- Custom dataset collected manually

- Annotated using Label Studio

- Annotation format: YOLO bounding boxes

- Total labeled images: 513

Dataset split:

- Training: 443 images (Approx. 86%)

- Validation: 70 images (Approx. 14%)

Classes include pedestrian-related scenarios relevant to zebra crossings.

Image Size and Preprocessing

- Original images vary in resolution and aspect ratio

- During training, images are automatically resized by YOLO

- Training image size parameter: `imgsz = 640`

- Aspect ratio is preserved internally by YOLO using padding (letterboxing)

This ensures consistent input dimensions while maintaining geometric correctness.

Training Setup

- Model: YOLOv11 nano (lightweight, embedded-friendly)

- Pretrained on COCO dataset

- Fine-tuned on custom pedestrian dataset

- Image size: 640 × 640

- Epochs: 100

- Batch size: 16

- Training script: train.py

- Dataset configuration: yolo_config.yaml

Evaluation

Model performance was evaluated using the standard metrics provided by the Ultralytics YOLO training pipeline.

Evaluation Metrics

- Precision (P): Measures the correctness of pedestrian detections

- Recall (R): Measures how many actual pedestrians were detected

- mAP@0.5: Mean Average Precision at IoU threshold 0.5

Training and Validation Outputs

The following evaluation artifacts were automatically generated during training:

- Loss curves

  - Box loss, classification loss, and DFL loss for training and validation

- Precision–Recall curves

  - BoxP_curve.png

  - BoxR_curve.png

  - BoxPR_curve.png

  - BoxF1_curve.png

- Confusion matrices

  - confusion_matrix.png

  - confusion_matrix_normalized.png

- Prediction visualizations

  - Training batches: train_batch*.jpg

  - Validation ground truth: val_batch*_labels.jpg

  - Validation predictions: val_batch*_pred.jpg

- Training summary

  - results.png

  - results.csv (numerical training statistics)

- Final trained weights

  - Best performing model: best.pt

These outputs confirm correct model convergence and allow qualitative and quantitative assessment of pedestrian detection performance on both training and validation datasets.

IMX500 Deployment:

The trained YOLO model is exported to IMX format for deployment on the Sony IMX500 AI vision sensor.

Export process:

- Convert `best.pt` to IMX format using `yolo_export.py`

- Generated output files:

  - `packerOut.zip`

  - `labels.txt`

- Convert exported model to RPK format for Raspberry Pi + IMX500 deployment

Deployment scripts are located in: IMX500/

Desktop Testing

The system can be tested in a desktop environment using:

- Python

- OpenCV

- Ultralytics YOLO framework

This fulfils the project requirement for desktop testability.

Limitations

- Reduced performance under low-light conditions

- Single-camera monocular setup

- No temporal tracking across frames

Possible Improvements

- Expand dataset with more environmental diversity

- Include night, foggy, and shadow conditions

- Add temporal tracking for video streams

- Combine with additional ADAS perception modules

Conclusion

This project demonstrates a complete end-to-end pedestrian detection pipeline,

from annotation and training to embedded deployment on IMX500 hardware.

All project requirements are satisfied, and the system provides a strong foundation

for automotive safety applications.

Author

Niraliben Yash Jani – 22402416

Satyajit Sushant Pardeshi - 22408966

Master’s Student – Automotive / AI Systems